100V Zener V1

This is a Simplis simulation model of a simple 100V zener diode. The ideal diode characteristics is simulated by a PWL resistor R6. The table is in V,I format. R1 is the series zener resistance.

The table can be modified to simulate any zener diode breakdown voltage. A softer knee characteristics can be simulated by adding more data points.